EE105: Introduction to Electrical Engineering

Diode I-V Characteristics

Objective:

In this lab, we’ll learn about Diodes, how they work, and their I-V characteristics. We will also cover DAC (digital-to-analog converter) basics and some initial discussion about the Differential Amplifier.

Diode Basics:

A diode is a two-terminal semiconductor device that allows current to flow primarily in one direction. It is formed by a p-n junction, exhibiting low resistance in forward bias and high resistance in reverse bias. Diodes are used for rectification, signal demodulation, voltage regulation, and protection.

I=I0(eqVnkT-1)

Figure-1: I-V Characteristics of a Diode.

The following setup will be demonstrated today to evaluate the I-V characteristics of a diode. (This setup is embedded in Module-2 of the development board.)

Figure-2: Diode I-V characteristics measurement setup.

Checkpoint-1: Select Module-2 on your dev board. Check module-2 and try to find different components of Figure-2 (i.e., diode, differential amplifier, DAC, etc.)

Digital-to-Analog Converter (DAC):

Since Arduino is a digital system, we’ll need DACs to convert digital signals from Arduino to analog signals. From this point forward, we’ll use DACs as analog supply/sweep voltage sources. AD5593R is the DAC module present on our development board. Please consult this datasheet for details.

Figure-3: DAC illustration.

Differential Amplifier:

A differential amplifier compares two input voltages and amplifies the difference between them.

Figure-4: Differential Amplifier using Op-Amp.

In this lab, we’ll use the differential amplifier to get the voltage across the 1K resistor that’s in series with our diode.

Checkpoint-2: Find out the amplification factor of the differential amplifier in Figure-2.

Diode I-V Characteristics using Arduino and Python:

In order to get the I-V characteristics of a diode, we need to sweep a voltage across the diode and measure the current through it. In our setup, we are using a DAC as the sweep voltage source. The Differential Amplifier, along with Arduino’s ADC, will help us read the current through the diode.

Checkpoint-3: Open diode> diode.ino in your Arduino IDE. Notice that, there are multiple files inside the diode folder, all of these header files are essential to make sure that the DAC module works properly.

Checkpoint-4: Identify and install any additional libraries in your Arduino IDE to run this code.

Checkpoint-5: Utilize m2_diode_discrete() function in diode.ino to determine currents for three discrete DAC voltage values, i.e., 0.25V, 0.9V, and 2.5V.

Checkpoint-6: Utilize m2_diode_sweep() function in diode.ino to perform a sweep of the DAC  voltage. There should be 3 columns of data in the serial monitor. Determine which quantities from Figure-2 each of these data columns represent.

Checkpoint-7: Copy the data from Serial Monitor onto a .csv file. Name this file to be ‘diode.csv’ and save in your work directory.

Checkpoint-8: Open diode.ipynb python notebook and plot the IV characteristics of the diode.

Checkpoint-9: From the IV curve, find the forward voltage (or the Knee voltage) of your diode.

Think about it: Suppose we are using a 0.5K resistor instead of a 1K resistor in series with our diode. Everything else about the circuit is still the same. What changes do you need to make in your Arduino/Python code to get the correct I-V curve of your diode?