EE105: Introduction to Electrical Engineering

Transistor I-V Characteristics

Objective:

In this lab, we’ll learn about transistors, how they work, and their I-V, or more accurately I-Vg-Vd characteristics. We will also cover DAC (digital-to-analog converter) basics and some initial discussion about the Transimpedance Amplifier.

Transistor Basics (NMOS, NPN):

An NMOS or NPN transistor is a three-terminal semiconductor device used for amplifying signals or as an electronically controlled switch. A small voltage at its gate (NMOS) or a small current into its base (NPN) controls a much larger current flow between the other two terminals. This control property makes transistors the fundamental building blocks for amplifiers and digital logic circuits.

Figure-1: I-V Characteristics of a NMOS Transistor.

The following setup will be demonstrated today to evaluate the I-V characteristics of a NMOS transistor. (This setup is embedded in Module-2 of the development board.)

Figure-2: Transistor measurement setup.

Checkpoint-1: Select Module-2 on your dev board. Check module-2 and try to find different components of Figure-2 (i.e., transistor, transimpedance amplifier, DAC, etc.)

Digital-to-Analog Converter (DAC):

Since Arduino is a digital system, we’ll need DACs to convert digital signals from Arduino to analog signals. From this point forward, we’ll use DACs as analog supply/sweep voltage sources. AD5593R is the DAC module present on our development board. Please consult this datasheet for details.

Figure-3: DAC illustration.

Transimpedance Amplifier:

A transimpedance amplifier (TIA) is a current-to-voltage converter, producing an output

voltage that is proportional to an input current.

Figure-4: Transimpedance Amplifier using Op-Amp.

In this lab, we’ll use the transimpedance amplifier to measure the current coming out of Source of the transistor.

Transistor I-V Characteristics using Arduino and Python:

In order to get the I-V characteristics of a transistor, we need to sweep a voltage through the gate or the drain of the transistor and measure the current through the source . In our setup, we are using a DAC as the sweep voltage source. The transimpedance amplifier along with Arduino’s ADC will help us read the current through the source.

Checkpoint-2: Open transistor.ino in your Arduino IDE. Alternatively you can use CloudeAgent with this sketch. Notice that there are multiple files inside the transistor folder/sketch, all of these header files are essential to make sure that the DAC module works properly.

Checkpoint-3: Identify and install any additional libraries in your Arduino IDE to run this code.

Checkpoint-4: Utilize discrete_read() function in transistor.ino to determine current at the source for at least three discrete DAC voltage pairs values, i.e., (0.25V, 0.9V), (1V, 2V), etc.

Checkpoint-5: Utilize voltage_sweeping() function in transistor.ino to perform a voltage sweep at the Gate and then at Drain. There should be 3 columns of data in the serial monitor for each sweeping. Determine which quantities from Figure-2 each of these data columns represent.

Checkpoint-6: Copy each data from Serial Monitor onto a .csv file. Name these files to be ‘transistor_vd_step.csv’ and ‘transistor_vg_step.csv’, respectively. Upload these files to your python directory or Colab.

Checkpoint-7: Open transitor.ipynb python notebook and plot the IV characteristics of the transistor using the data from both voltage sweep, respectively.

Checkpoint-8: From the IV curve, find the saturation voltage Vdsat and threshold voltage Vt (refer to Figure 1) of your transistor.