EE105: Introduction to Electrical Engineering

LiFi Module

Objective:

In this lab, we’ll learn about Lifi Module and how to transfer data over this module.

What is LiFi?

- We have developed a code to demonstrate Li-Fi (Light Fidelity) technology, a form of free-space communication that transmits data using visible light.

- Li-Fi Technology: It utilizes light to transmit data wirelessly, typically through LEDs as transmitters and light sensors as receivers.

- Free Space Communication: This means the communication happens without any physical medium like cables; instead, it relies on light traveling through the air.

LED (Light Emitting Diode):

- Functions as the transmitter (Tx module).

- It emits light that is modulated (turned on and off rapidly) to encode the data.

- The modulation happens so quickly that the human eye cannot detect the blinking

PD (Photodiode):

- Functions as the receiver (Rx module).

- The Photodiode (PD) is used to detect changes in light intensity caused by the transmitting LED.

- The PD converts these light changes back into electrical signals, which are then interpreted as data.

How LiFi works:

Transmission:

- Data (e.g., a text string, binary signal) is encoded as light pulses.

- The LED blinks on and off in a specific pattern that corresponds to the transmitted data.

Reception:

- The LDR detects the changes in light intensity and converts them into an electrical signal.

- The Arduino processes the received signal to decode the data.

LiFi Transmission (lifi_tx and send_byte):

Purpose:

- The lifi_tx function sends data (e.g., a text string) through light modulation using an LED.

Process:

- A string (e.g., "This is a test transmission!") is split into individual characters.

- Each character is converted into an 8-bit binary representation.

- The send_byte function modulates the LED's brightness to represent each bit of the character:

  - HIGH (LED ON): Represents 1.

  - LOW (LED OFF): Represents 0.

- After transmitting all the bits of a character, a small delay ensures the receiver can distinguish between consecutive bytes.

Bit Transmission:

- Each byte is sent bit by bit, where the LED is turned ON or OFF depending on the bit value.

LiFi Reception (lifi_rx, get_byte, read_pd):

Purpose:

- The lifi_rx function receives the modulated light signal using a Photo Diode (PD) and decodes the transmitted data.

Process:

- The Photo Diode (PD) is used to detect changes in light intensity caused by the transmitting LED.

- The read_pd function monitors the PD value and determines whether it corresponds to a HIGH (1) or LOW (0) bit based on a threshold (THRESHOLD).

Printing Received Data:

- The reconstructed byte is converted back to its character form and displayed using the print_byte function.

Example:

LiFi in Action: Sending ‘Hi’:

The string "Hi" becomes:

  - 'H' → 72 → 01001000

  - 'i' → 105 → 01101001

  - Photo Diode detects light changes.

  - Arduino decodes the binary into 'H' and 'i’.

Conversion Process:

- Text is converted into numeric ASCII codes.

- ASCII codes are stored/transmitted as binary.

- Universal Standard:

  - ASCII was one of the first widely adopted character encoding systems,

ensuring consistency in how text is represented across different devices and systems.

Task-1:

- First, form pairs and position your boards next to each other such that the Photodiode (PD) and LED face each other.

- Run the devboard_sketch code from Task1 folder. Then, navigate to Tools > Serial Monitor and type the lifi_tx command to transmit data. Use the lifi_rx command to receive data.

- You should be able to see the message: “This is a test transmission.”

Task-2:

In the uploaded Task2 folder, you will find a file named message_small.txt. Your task is to transmit this message from one development board to another.

Folder Structure:

- /devboard_sketch_receiver: contains the Arduino code for the receiver

- /devboard_sketch_sender: contains the Arduino code for the sender

- sender.ipynb: Python notebook for communicating with the sender Arduino

- receiver.ipynb: Python notebook for communicating with the receiver Arduino

Steps:

- Upload the Arduino code to each board (sender and receiver) using the provided sketches.

- Use the Python notebooks (sender.ipynb and receiver.ipynb) to communicate with the boards. Install necessary python modules (i.e., pyserial, numpy, matplotlib, ipykernel etc.)

Goal:

In the actual challenge, you will transmit three different messages of varying lengths. Your objective is to achieve an accuracy > 95%